英翻中（五題、每題20分、共100分）

一、Since you replace brake pads in pairs, we are going to be doing both front brake pads, so I like jacking the car from the middle so both tires raise up at the same time. And the best way to lift up both front tires is by jacking it up from the front crossmember. This is a thick piece of metal and supports the whole front suspension.

二、And with the car in the air slide your jack stand underneath the car and I try to just stick my arms underneath the car when I moved the jackstand. Don't put your whole body under the car. You don't want to be under the car until the car is securely set onto the jack stands. Now we're looking for a place to put your jack stands.

三、And the last thing I like to do is give the car a good shake and make sure that it's stable and doesn't move. And with the car safely lifted off the ground we could remove the lug nuts and remove the wheel. And another thing I like to do for extra safety is slide that wheel underneath the frame. That way in worst-case scenario you have something solid there and the car won't drop all the way down.

四、Normally these bolts use a regular socket but in this case we have a torx bolt, so we're using a T50 torx. And I start off with a breaker bar every time just so you can break these bolts loose easily. And then once you use your breaker bar to break the bolt loose, you can use a regular ratchet to loosen it up the rest of the way and it'll come right out. And you can see the blue loctite on the thread so we'll be sure to add some when we screw this in later on.

五、And with the caliper removed you don't want the caliper to just dangle by the brake line because that's how you damage your brake line. So, instead, you want to try to find a place to put your caliper where it's out of the way and there's no pressure on the brake line. If you don't have a nice place to leave the brake caliper like that you could also use a bungee cord or rope or something to hang it up.