英翻中（五題、每題20分、共100分）

一、Since you replace brake pads in pairs, we are going to be doing both front brake pads, so I like jacking the car from the middle so both tires raise up at the same time. And the best way to lift up both front tires is by jacking it up from the front crossmember. This is a thick piece of metal and supports the whole front suspension.

二、And with the car in the air slide your jack stand underneath the car and I try to just stick my arms underneath the car when I moved the jackstand. Don't put your whole body under the car. You don't want to be under the car until the car is securely set onto the jack stands. Now we're looking for a place to put your jack stands.

三、Now we could pry open our caliper which might take a little force, and then remove our old brake pads and they'll come right out just like that. So for this brake caliper we just had to remove that one bolt right down here, but on other brake calipers there's also another bolt right up here that you could remove to take this whole clamshell off.

四、And with the caliper removed you don't want the caliper to just dangle by the brake line because that's how you damage your brake line. So, instead, you want to try to find a place to put your caliper where it's out of the way and there's no pressure on the brake line. If you don't have a nice place to leave the brake caliper like that you could also use a bungee cord or rope or something to hang it up.

五、Next, we want to remove this caliper bracket so we could pull the old brake rotor off, and to remove the caliper bracket we want to go behind the knuckle and you can see there's a bolt right up here and a bolt right down here holding this bracket in, that we need to remove, In this case it's a 15mm bolt, and again start off with your breaker bar so you can easily break these loose. These are going to be the tightest bolts, so they're going to be the toughest to break loose and again after we're done cracking the bolts loose with our breaker bar, we could loosen them the rest of the way with are shorter ratchet.