字義測驗 每題5分，共100分，答案請直接填入次頁答案欄內

- hey guys Chris fix here and today I'm going to show you how to replace every single fluid in your vehicle so whether you have a luxury car a muscle car a pickup truck an SUV a normal passenger car a compact car or a      convertible after you're done watching this video you're gonna know how to replace every fluid in your car at home with common hand tools this video will cover how to change engine oil, transmission fluid,  power steering fluid, engine coolant, brake fluid, rear differential fluid, transfer case fluid, washer fluid and finally we're going to force out all the old grease and add brand new grease to all the grease points on our suspension and if you're wondering how often you should replace your fluids and what type of fluid and how much you need we'll grab your owner's manual and let me show you now we're going to want to head to the back to the maintenance and specifications section and as you can see it lists the different types of fluids in your vehicle the specific fluid you should use and also the capacity the amount of fluid that you're going to need and some owner's manuals even have a page like this that it tells you how many miles that you need to change specific fluids at so it gives you the interval for example we have the oil right here it tells you to change it every 5 000 miles and then the coolant right there it tells you to replace every 45 000 miles so this will give you a good idea on when you should replace your fluids all right

二、 So now let's get started replacing all the fluids and the first thing we need to do is lift up the vehicle if you have a lift that's awesome definitely use it in this case we don't have a lift we have a jack and jack stands and that's what I'm going to be using we're going to lift the front and then we're going to lift the rear so let's get started first thing you want to do is chalk off the rear wheels and we're going to pull the e-brake so the car doesn't move next let's slide the jack under the front of the car and we want to jack up under the cross member which could easily support the weight of the car as we lift it so jack the car up as high as you can so we can get under and change all the fluids comfortably now you want to grab your jack stand and look underneath the vehicle for the frame rail and to find the frame rail on your car just look for this large metal piece that runs from the front all the way down to the back so get the jack stand underneath the frame rail like that and I have our other jack set up as well then we can slowly lower the car onto the jack stands and remove the jack from the front so with the front of the car safely lifted up now we're gonna come to the back slide the jack under the car and back here we're gonna lift it up from the rear differential just make sure the jack is centered on the differential so it doesn't slide off as you lift it and as you jack up the car keep an eye on the front jack stands to make sure they're not tilting or sliding and as we lift up the rear you want to get it lifted up high enough so the car is level just like that so at the car level in the rear we need to find a spot to put the jack stand this right here is metal on the frame but I don't think it's thick enough I'd rather get something thicker and a little bit sturdier there is a giant piece of subframe metal right there that is thick and sturdy and that'll be safe so let's slide the jack underneath the car get it under that thick metal subframe and then let's slowly lower it down onto the jack stands just like that and I like to keep the jack resting up slightly against the differential as extra protection just in case the jack stand fails so there we go now the car is completely lifted off the ground and anytime I lift the vehicle I like double protection so in the back here not only do we have our jack stands but we have the jack holding up the differential just in case and if we come over to the front not only do we have the jack stands under the frame rails but we have ramps underneath the tires you could use ramps you could use wheels whatever you have just have double protection two things to prevent the car from falling on you.

字義測驗

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| luxury car | pickup truck | SUV | compact car | convertible | washer fluid | grease | suspension | owner's manual | lift |
| 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| e-brake | cross member | frame | tilting | sturdier | double protection | jack stands | differential | ramps | prevent |